# Universty Network Design

Session: 2024 – 2028

Submitted by:

Ahmad Hassan Tanseel     2024-CS-811

Supervised by:

Mr. Muhammad Tahir

Subject :

Computer Networks (CS-203)

Department of Computer Science

University of Engineering and Technology, Lahore

Pakistan.

# University Network Design and Implementation Report

## 1. Introduction

This report presents the planning, design, implementation, and evaluation of the network infrastructure for University, which operates across two campuses located 20 miles apart. The university accommodates four faculties: Health and Sciences, Business, Engineering/Computing, and Art/Design. Both staff and students require reliable, secure, and scalable network connectivity with access to internal and external services.

The network was designed and implemented using Cisco Packet Tracer, following modern networking principles including VLAN segmentation, dynamic IP addressing, RIPv2 routing.

## 2. Network Planning and Design

### 2.1 Network Design Objectives

The main objectives of the proposed network design were:

- Logical separation of faculties and departments

- Secure and efficient communication between campuses

- Centralized routing and server management

- Scalability for future expansion

- High availability and ease of maintenance

### 2.2 Campus Overview

#### Main Campus

- Building A

  - Management Department

  - HR Department

  - Finance Department

  - Faculty of Business

- Building B

  - Faculty of Engineering & Computing

  - Faculty of Art & Design

- Building C

  - Student Computer Labs

  - IT Department

  - Internal Servers (Web, File, DNS, etc.)

#### Branch Campus

- Faculty of Health & Sciences

- Separate floors for staff and student labs

### 2.3 Logical Network Segmentation (VLAN Design)

Each department and faculty was placed on a separate VLAN and IP network to enhance:

- Security

- Broadcast control

- Network performance

| VLAN | Department / Faculty |
| --- | --- |
| VLAN100 | Lab |
| VLAN 10 | Management |
| VLAN 20 | HR |
| VLAN 30 | Finance |
| VLAN 40 | Business |
| VLAN 50 | Engineering & Computing |
| VLAN 60 | Art & Design |
| VLAN 70 | Student Labs |
| VLAN 80 | IT Department |
| VLAN 90 | Health & Sciences |

### 2.4 IP Addressing Scheme

A Class C private addressing scheme was used:

Example:

- 192.168.10.0/24 – Management

- 192.168.20.0/24 – HR

- 192.168.30.0/24 – Finance

- 192.168.40.0/24 – Business

- 192.168.50.0/24 – Engineering

- 192.168.60.0/24 – Art & Design

- 192.168.70.0/24 – Student Labs

- 192.168.80.0/24 – IT Department

- 192.168.90.0/24 – Health & Sciences

## 3. Network Configuration and Implementation

### 3.1 Network Topology Setup

The topology includes:

- Core routers at each campus

- Layer-2 switches for access

- Trunk links between switches

- WAN link connecting campuses

- External cloud-based email server

### 3.2 VLAN Configuration on Switches

Each switch was configured with:

- VLAN creation

- Access port assignment

- Trunk ports

### 3.3 Inter-VLAN Routing

Router-on-a-Stick configuration was used:

- Sub-interfaces for each VLAN

- Encapsulation dot1Q

- Gateway IP for each VLAN

### 3.4 DHCP Configuration (Building A)

A router-based DHCP server was implemented to dynamically assign IP addresses to:

- Management

- HR

- Finance

- Business departments

DHCP pools were created for each VLAN with:

- Network address

- Default gateway

- DNS server

### 3.5 Routing Protocol Configuration

- RIPv2 was used for internal routing between routers

- Automatic route updates enabled

- Version 2 selected to support CIDR

- Static routing configured for the external email server

### 3.6 External Email Server (Cloud)

The email server is hosted externally:

- Static route configured on edge router

- Successful ICMP and application-level connectivity verified

### 3.7 End-to-End Connectivity Testing

Connectivity tests performed:

- Ping between VLANs

- Access to internal web server

- Access to external email server

- DHCP verification

## 4. Evaluation and Critical Appraisal

### 4.1 Performance

### The network demonstrates strong performance due to:

- VLAN-based broadcast reduction

- Efficient routing using RIPv2

- DHCP

Latency between campuses remains acceptable despite distance.

### 4.2 Scalability

The design is highly scalable:

- New departments can be added via VLANs

- IP addressing supports expansion

- Additional switches and routers can be integrated easily

### 4.3 Reliability

Reliability is ensured through:

- Structured hierarchical design

- Centralized IT management

- Dynamic routing adaptation

### 4.4 Security

Security measures include:

- VLAN isolation

- Limited broadcast domains

- Controlled routing paths

Further improvements could include:

- ACLs

- Firewall implementation

- Port security

## 5. Conclusion

This project successfully delivered a fully functional, secure, and scalable network for University. The use of VLANs, RIPv2, DHCP, and structured design ensures that both campuses can operate efficiently while meeting academic and administrative needs.The network satisfies all stated requirements and provides a strong foundation for future growth and enhancement.

## 6. References

- Cisco Networking Academy Documentation

- Cisco Packet Tracer User Guide

- Computer Networking Principle

- YouTube